Digital Watchdog is a leading manufacturer of security and surveillance solutions, offering stunning image quality, advanced hardware capabilities, reliable customer support and the lowest total cost of deployment to the analog & IP megapixel surveillance markets. Located in Tampa, FL with manufacturing facilities in Seoul, Korea, Digital Watchdog is committed to delivering powerful security solutions to its customers worldwide.

For additional information, contact:

- Digital Watchdog®

- 16220 Bloomfield Avenue.

- Cerritos, California USA 90703 USA

- Phone: (866) 446-3595

- Web: www.digital-watchdog.com

- E-mail: dw-tech@digital-watchdog.com

MEGApix® PANO™ 21MP PANORAMIC VANDAL IP CAMERA

- DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

Important:  See further notes on the following page.

- Important Note to Security Systems Specifiers

- CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

- Primary Specification Area:

- MasterFormat 2014:

28 20 00 	Electronic Surveillance

28 23 00 	Video Surveillance

28 23 29	Video Surveillance Remote Devices and Sensors

- MasterFormat 2016:

- 28 20 00	Video Surveillance

- 28 2x xx	Surveillance Cameras

- 28 2x xx	IP Cameras

- 28 2x xx	Panoramic IP Cameras

- Related Requirements:

- MasterFormat 2014:

- 27 20 00 Data Communications

- 28 23 13 Video Surveillance Control and Management Systems

- 28 23 16 Video Surveillance Monitoring and Supervisory Interfaces

- 28 23 19 Digital Video Recorders and Analog Recording Devices

- 28 23 23 Video Surveillance Systems Infrastructure

- MasterFormat 2016

- 27 15 01.xx	Video Surveillance Communications Conductors and Cables

- 27 20 00 	Data Communications

- 28 05 xx.xx	PoE Power Sources for Electronic Safety and Security

- 28 05 xx	Storage Appliances for Electronic Safety and Security

- 28 05 xx.xx	Network Video Recorders

- 28 05 xx	Cyber Requirements for Electronic Safety and Security

- 28 05 xx	Safety and Security Network Communications Equipment

28 2x 00	Video Management System

MEGApix® Pano™ 21MP PANORAMIC VIEW VANDAL IP CAMERA

- GENERAL

  - SUMMARY

    - The section includes a high-resolution 180o panoramic camera suitable for outdoor deployment.

    - Product - A four-sensor, 21MP panoramic dome camera, with dual streaming H.264 and MJPEG capability in a vandal resistant IP66 rated dome housing and capable of producing multiple zoom streams from a single master IP stream.

    - Related Requirements

      - 27 15 01.13 – Video Surveillance Communications Conductors and Cables

      - 28 05 03 - Safety and Security Network Communications Equipment

      - 28 05 19 – Storage Appliances for Electronic Safety and Security

      - 28 06 20 - Schedules for Video Surveillance

      - 28 23 00 - Video Management System

  - REFERENCES

    - Abbreviations

      - AGC - Automatic Gain Control

      - APIPA - Automatic Private IP Addressing

      - ARP – Address Resolution Protocol

      - AWB  - Automatic White Balance

      - BLC – Back light compression

      - DHCP - Dynamic Host Configuration Protocol

      - DNR – Digital Noise Reduction

      - DNS - Domain Name Server

      - fps - frames per second

      - FTP - File Transfer Protocol

      - GOP – Group of Pictures

      - GUI – Graphical User Interface

      - HLC – Highlight Compensation

      - HTTP - Hypertext Transfer Protocol

      - ICMP – Internet Control Message Protocol

      - IGMP - Internet Group Management Protocol

      - IP - Internet Protocol

      - JPEG - Joint Photographic Experts Group

      - MJPEG - Motion JPEG

      - MP - Megapixel

      - MPEG - Moving Pictures Experts Group

      - NTP - Network Time Protocol

      - PoE - Power over Ethernet

      - RTP - Real-Time Transport Protocol

      - RTSP - Real-Time Streaming Protocol

      - SMTP - Simple Mail Transfer Protocol

      - SNMP – Simple Network Management Protocol

      - TCP - Transmission Control Protocol

      - UDP - User Datagram Protocol

      - UPnP – Universal Plug and Play

      - VMS - Video Management System

      - WDR – Wide Dynamic Range

      - ZeroConf – Zero Configuration Networking

    - Reference Standards

      - Network

        - IEEE - 802.3 Ethernet Standards

      - Video

        - ISO / IEC 14496–10, MPEG-4 Part 10 ( ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF – Profile S

      - Emissions

        - FCC-47 CFR Part 15 Class A

        - EN55022: 1998 with A1: 2000, A2: 2003; Class A

        - CISPR 22: 1997 with A1: 2000, A2: 2002; Class A

      - Noise Immunity

        - EN 55024: 1998 with A1: 2001, A2: 2003

        - CISPR 24: 1997 with A1: 2001, A2: 2002

      - Safety

        - EN 60950-1: 2001 with A11: 2004

        - IEC 60950-1: 2001

      - Environmental

        - ANSI / IEC60529 – Degrees of Protection Provided by Enclosures – IP66

    - Definitions

      - Virtual Camera – Operation deriving multiple scenes from one IP video stream, creating the appearance of multiple cameras.

      - Zero Configuration Networking - is a set of technologies that automatically creates a usable computer network based on the Internet Protocol Suite (TCP/IP) when peripherals are interconnected, without requiring manual operator intervention or special configuration servers

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

      - Warranty documentation

  - QUALIFICATIONS

    - The manufacturer shall have a minimum of five years of experience in producing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment of 22°F to 122°F (-4°C to 50°C), protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY AND SUPPORT

    - The manufacturer shall provide a 5-year warranty for the product to be free of defects in material and workmanship.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Digital Watchdog®

16220 Bloomfield Avenue.

Cerritos, California USA 90703 USA

Phone: (866) 446-3595

Web: www.digital-watchdog.com

E-mail:  dw-tech@digital-watchdog.com

    - Model		DWC-PZ21M69T

Digital Watchdog model differences:

DWC-PZ21M69T	Silver-color housing

DWC-PZ21M69TW	White-color hosing

    - Alternates: 		None

  - GENERAL DESCRIPTION

    - The 21MP Panoramic Weather-Resistant Camera (“panoramic camera”) shall employ four 6.4MP CMOS sensors to produce 180° views at up to 30fps at 21MP resolution via a single IEEE 802.3at compliant connection.

    - A single IP video stream from the panoramic camera shall be able to produce multiple zoom streams with virtual camera operation.

    - The panoramic camera shall be housed in a vandal-resistant IP-66 rated weatherproof enclosure with zero distortion anti-reflective and 98%+ transmissive glass, suitable for outdoor deployment.

    - Each lens in the panoramic camera shall be motorized allowing for remote control of rotation and focus and shall have the ability to be tilted to accommodate specific heights and distances from the target.

    - The panoramic camera shall be ONVIF Profile S compliant.

    - The panoramic camera shall possess the following further characteristics:

      - H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

      - Day/night operation with movable IR cut filters

      - Low light level operation to 0.16 lux (color)

      - 3D digital noise reduction

      - Built-in web server

      - 4x Micro SD/SDHC/SDXC

      - Integral motion detection

      - Simultaneous access by up to 10 users

      - Multicast or unicast capable

  - VIDEO

    - Imager

      - Sensors (4):			1/1.8" 6.4 MP Sony CMOS

        - pixels per sensor:		3096 (H) x 2080 (V)

      - Minimum illumination

        - Color mode: 		0.16 lux

      - Image Control Settings shall be available for:

        - Automatic white balance (AWB)

        - Exposure modes: 		automatic or manual

          - In manual exposure mode, configurable settings for:

            - shutter mode: 	automatic or manual - 1/15 to 1/32000

            - slow speed:		1/2 , 1/3 , 1/5 , 1/6 , 1/7.5 , 1/10

            - backlight compensation (BLC)

        - Day and night settings, allow configuration for Day (color), Night (Black and White), or Automatic.

        - Image mirror or flip

        - Digital noise reduction

      - Lenses (4):			8.0 mm fixed

        - Field of view:		>180 o

        - Manual functions:

          - focus

          - IR filter operation

        - The lenses shall have the option for autofocus which will locate the sharpest image setting averaged over the entire individual scene and maintain lens focus position even after reboot

    - Video Streams

      - The panoramic camera shall support two configurable video streams, each of which may have the following properties:

        - Resolution:			up to 3096 x 2080

        - Streams:

          - Number:		independently configurable

          - Frames per second:	configurable, up to the maximum specified resolution

          - GOP size:		user-controllable

        - Compression type:

          - H.264

          - MJPEG, with controllable quality

        - Bit rate:			100Kbps ~ 10 Mbps, controllable for minimum and maximum

        - Frame rates vs. resolution:

          - 21MP			30fps

    - Storage and Recording

      - The panoramic control shall have onboard SD card storage.

        - Card type:	4x Micro SD/SDHC/SDXC

      - Local recording on the SD card shall commence upon loss of network connectivity, based on a pre-programmed schedule.

      - The local SD storage shall have the ability to be backed up to alternate media without the removal of the SD card from the camera.

    - Video streams shall be capable of supporting ONVIF protocol, profile S.

    - Other

      - Discovery - The manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

      - Access - The panoramic camera shall permit up to ten users to simultaneously access the camera.

  - AUDIO

    - The IP dome camera shall provide one-line level input and one-line level output.

    - Compression and Sampling Rate:  G.711 (8 kHz)

  - NETWORK

    - Connectivity: 	100/1000 Base-T Ethernet via RJ-45 connector

    - Protocols supported

      - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

      - Configuration: Dynamic Host Configuration Protocol (DHCP)

      - Web services: Hypertext Transfer Protocol (HTTP), Secure HTTP (HTTPS)

      - Network services: Domain Name System (DNS), Network Time Protocol (NTP), Internet Control Message Protocol (ICMP), Simple Network Management Protocol (SNMP)

      - Media: Real-Time Transport Protocol (RTP), Real-Time Streaming Protocol (RTSP)

      - Notifications: File Transfer Protocol (FTP), Simple Mail Transfer Protocol (SMTP), Telnet

    - Flow Control. The panoramic camera shall support TCP and Layer 3 QoS protocols to control network congestion.

    - DDNS – The Panoramic camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

    - Security

      - The panoramic camera shall support IP address filtering whereby users can enter a list of allowed or blocked IP addresses for viewing video and configuring camera settings

      - The panoramic camera shall provide three levels of user access with password protection.

  - ADDITIONAL FEATURES

    - Auxiliary Inputs and Outputs

      - Input: Alarm sensor (contact closure)

      - Output: Relay

    - System Information

      - The system settings of the panoramic camera shall be exportable as a separate file.

      - The panoramic camera shall maintain an accessible log of system and motion-triggered events.

        - The log shall be exportable to an Excel spreadsheet file.

  - CAMERA SOFTWARE

    - The panoramic camera shall have a built-in web server that supports browser-based configuration using Internet Explorer, Google Chrome, Mozilla Firefox

    - The software GUI shall allow access to camera information and all primary software functions, including:

      - Camera network address

      - Configuration

      - Stream Control

      - Screen snapshot, print and export

      - Start and stop recording

    - The Manufacturer shall offer video viewer and configuration tools to implement the following actions:

      - Camera discovery

      - Configuration and control

        - imager:

          - active noise filter

          - localized exposure

          - back-light compensation

          - exposure control for ac lighting anti-flicker

          - advanced exposure settings

            - metering mode

            - metering area

            - exposure target factor

            - min and max shutter time settings

            - definable sensors gain control

          - focus control

          - day/night settings

        - image:

          - saturation

          - brightness

          - hue

          - manual contrast

          - sharpness

          - auto digital WDR

          - auto contrast

          - white balance

        - events and notifications

          - motion related

        - camera network parameters

        - SD card storage recording management

        - image capture, export, and print

      - Viewer - view video streams through the web browser

      - Image print and export

      - Instant recording and playback

      - Alerts

        - e-mail setup

        - define web addresses for notifications

      - System

        - firmware upgrade

        - reset to factory default

        - set date, time, and NTP server synchronization

        - user access control

        - View and export camera settings

        - view system logs

    - The panoramic camera shall be supported by Video Management Systems from the Manufacturer and third-party manufacturers.

  - ELECTRICAL

    - Power

      - Sources

        - 12 VDC

        - PoE:			IEEE802.3at, PoE+ class 4, PoE Injector included

      - Power Consumption:		12VDC: 21.5W

PoE+: 23.2W

      - Ethernet: 			RJ-45 connector

      - External power (12 VDC): 	2-wire pigtail

  - MECHANICAL AND ENVIRONMENTAL

    - Material:

      - Housing:			aluminum diecast, vandal resistant

      - Protective lens windows:	anti-reflective glass

    - Configuration: 			panoramic vandal camera

    - Dimensions (D x H):  		7.55 in. x 4.25 in. (192 mm x 102 mm)

    - DWC-PZ21M69T Mounting Options:

      - Ceiling Mount Bracket		DWC-PZCM

      - Wall Mount Bracket		DWC-PZWM

      - Corner Mount			DWC-V1CNM (works with DWC-PZWM)

      - Adapter for Parapet Mount	DWC-PZADP

      - Parapet Mount 			DWC-PZPARAM (DWC-PZADP required, sold separately)

      - Pole Mount Bracket		DWC-PMB-WL (works with DWC-PZWM)

    - DWC-PZ21M69TW Mounting Options:

      - Ceiling Mount Bracket		DWC-PZCMW

      - Wall Mount Bracket		DWC-PZWMW

      - Corner Mount			DWC-V1CNMW (works with DWC-PZWMW)

      - Adapter for Parapet Mount	DWC-PZADPW

      - Parapet Mount 			DWC-PZPARAMW (DWC-PZADPW required, sold separately)

      - Pole Mount Bracket		DWC-PMB-WLW (works with DWC-PZWMW)

    - Temperature:

      - Operating

        - standard:			4°F ~ 122°F (-22°C ~ 50°C)

    - Humidity:				0 - 80%, non-condensing

    - Environmental Rating:		IP66

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor before camera installation.

  - INSTALLATION

    - The contractor shall follow all Manufacturer issued instructions for the installation of the product.

    - Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment

      - A report indicating successful test results shall be produced.

  - STORAGE

    - The panoramic camera hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

  - ATTACHMENTS

    - Supported Third Party VMS Systems

END OF SECTION