Digital Watchdog® is a leading manufacturer of security and surveillance solutions, offering stunning image quality, advanced hardware capabilities, reliable customer support and the lowest total cost of deployment to the analog & IP megapixel surveillance markets. Located in Cerritos, CA, with manufacturing facilities in Seoul, Korea, Digital Watchdog® is committed to delivering powerful security solutions to its customers worldwide.

For additional information, contact:

- Digital Watchdog®
	16220 Bloomfield Avenue,

- Cerritos, California 90703 USA

- Phone: +1 888 446-3593

- Web: www.digital-watchdog.com

- Email: dw-tech@digital-watchdog.com

DW SPECTRUM® CLOUD-BASED VIDEO MANAGEMENT SERVICES

- DIVISION 28 – ELECTRONIC SAFETY AND SECURITY (MasterFormat 2016)

28 20 00 	Video Surveillance

28 23 13	Video Surveillance Management System

Notes to Specifier:

1. Where several alternative parameters or specifications exist or where the specifier has the option of inserting text, such choices are presented in <bold text>.

2. Explanatory notes and comments are presented in colored text.

3. CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although the adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

4. MasterFormat 2014 Specification Category:

28 23 13 - Video Surveillance Control and Management Systems

DW SPECTRUM® CLOUD-BASED VIDEO MANAGEMENT SERVICES

- GENERAL

  - SUMMARY

    - The section includes an IP Video Management System.

    - Product - DW Spectrum is an elegantly easy cutting-edge approach to video surveillance, addressing the primary limitations of managing enterprise-level HD video while offering the lowest total cost of deployment and ownership on the market. DW Spectrum includes DW Cloud™ to easily manage all your systems and users from a simple webpage. DW Spectrum is a perfect solution for applications that require ease of use, speed, efficiency and unprecedented image quality.

    - General Requirements

      - The specified product shall be an open, extensible video platform designed for use in any video application.

      - The specified software shall include, free of charge, any API or SDKs necessary to integrate 3rd party devices and systems.

      - The specified Video Management solution's architecture should include Desktop, Server, Mobile, and Cloud applications.

    - Related Requirements

      - 28 05 03 - Safety and Security Network Communications Equipment

      - 28 05 15 - Servers for Electronic Safety and Security

      - 28 05 17 - Workstations for Electronic Safety and Security

      - 28 06 20 - Schedules for Video Surveillance

      - 28 21 00 - Surveillance Cameras

  - REFERENCES

    - General Abbreviations, acronyms, and standards

      - ACC: Video Codec "Active Content Compression

      - ADDS: Active Directory Domain Services

      - AGC: Automatic gain control

      - API: Application Programming Interface

      - AVI: Audio Video Interleave

      - Bit Rate: The number of bits/time units sent over a network

      - DHCP: Dynamic Host Configuration Protocol

      - FPS: Frames per Second

      - FTP: File Transfer Protocol

      - GbE: Gigabit Ethernet (1000Mbps)

      - 264/5 (Video Compression Format)

      - HTTP: HyperText Transport Protocol

      - IEEE 802.1x: Authentication framework for network devices

      - IP: Internet Protocol

      - JPEG: Joint Photographic Experts Group (image format)

      - LAN: Local Area Network

      - MJPEG: Motion JPEG

      - MKV: Matroska video format

      - MP4: MPEG Layer-4 Audio

      - MPEG: Moving Picture Experts Group

      - NTP: Network Time Protocol

      - NTSC: National Television System Committee – a color encoding system based on 60Hz

      - ONVIF: Global standard for the interface of IP-based physical security products

      - PoE: Power over Ethernet (IEEE 802.3af/at) standard for providing power over network cable

      - PTZ: Pan/Tilt/Zoom

      - SDK: Software Development Kit

      - RAID: Redundant Array of Independent Disks

      - RTSP: Real-Time Streaming Protocol

      - RADASS: Resolution and Algorithmic Data Adaptive Scaling System

      - SMTP: Simple Mail Transfer Protocol

      - SSL: Secure Sockets Layer

      - TCP: Transmission Control Protocol

      - TLS: Transport Layer Security

      - Unicast: Communication between a single sender and a single receiver on a network

      - VMS: Video Management System

    - Definitions

      - E-mapping – Images that can be used as background in the client software to show server and camera placement at a site.

      - Media Server – A server/controller for cameras and network connections, including client connections and camera stream encoding for hard disks.

      - Motion sensitivity – Degree of pixelization change in a selected area of the video scene required to trigger a recording or alarm.

      - Power Users – A collective user group comprised of users equipped with Administrator rights and access in the system, including camera settings and system settings.

    - Reference Standards

      - Network

        - IEEE - 802.3 Ethernet Standards

      - Video

        - ISO / IEC 14496 – MPEG-4

          - ISO / IEC 14496–10, MPEG-4 Part 10 (ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF – Profile S

  - Quality Assurance

    - Basic Level of Support

      - Complete product and technical data specification sheets that include all material and equipment shall be provided by the System Integrator and be available freely online.

  - SUBMITTALS

    - Basic Level of Support

      - Complete product and technical data specification sheets that include all material and equipment used on this project shall be included in the submitted solution proposal.

    - Product Documentation

      - Complete product and technical data specification sheets that include all material and equipment used on this project shall be included in the submitted solution proposal.

    - System Documentation

      - The System Integrator will provide:

        - List all equipment with part numbers, Manufacturer, firmware, and assigned IP addresses.

        - Locations and details for all components to be installed under this scope of work.

    - System Planning

      - Placement Diagram - The System Integrator will provide a placement diagram showing the proposed location of all system hardware devices.

      - System Calculation - The System Integrator will provide a calculation of all network bandwidth and storage requirements for System Servers to ensure proper planning of computing and networking infrastructure.

    - Qualifications

      - The Manufacturer shall have a minimum of five (5) years' experience in producing IP video equipment and software.

      - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, commission, and provide ongoing support for the solution.

  - ONGOING SUPPORT AND WARRANTY

    - Integrator Warranty

      - The security system DW Spectrum software and labor furnished by the Integrator, including wiring, software, hardware and third-party products, shall be fully warranted for parts, materials and labor for a minimum of 1 year from the date of the final acceptance of the Video Surveillance System.

    - Software Licensing and Warranty

      - Software licensing should be on a per-device basis (e.g., 1 x license for 1 IP Camera or I/O device) with no base license for additional features or capabilities.

      - The DW Spectrum Software should be completely free for discovering compatible IP cameras and devices, as well as live streaming or playback of offline media files (images, videos).

      - The Manufacturer shall provide software upgrades without cost and the need for an annual maintenance agreement.

  - SUPPORT

    - The Manufacturer shall provide lifetime software upgrades without cost and annual maintenance agreements.

END OF SECTION

- PRODUCT OVERVIEW

  - EQUIPMENT

    - Manufacturer: 	Digital Watchdog, Inc.

16220 Bloomfield Avenue. Cerritos,

California USA 90703 USA

Phone: (866) 446-3595

Web: www.digital-watchdog.com

Email:  dw-tech@digital-watchdog.com

    - Models: DW-SPECTRUMLSC

    - Alternates:

      - DW Spectrum licenses

        - DW-SPECTRUMLSC001	Single (1) DW Spectrum recording license

        - DW-SPECTRUMLSC004	Four (4) DW Spectrum recording licenses

        - DW-SPECTRUMLSC010	Ten (10) DW Spectrum recording licenses

        - DW-SPECTRUMLSC020	Twenty (20) DW Spectrum recording licenses

        - DW-SPECTRUMLSC050	Fifty (50) DW Spectrum recording licenses

        - DW-SPECTRUMLSC100	One Hundred (100) DW Spectrum® recording licenses

      - DW Spectrum analog-to-IP encoder licenses

        - DW-SPCP04LSC004	Four (4) DW Spectrum analog-to-IP encoder licenses

        - DW-SPCP16LSC016	Sixteen (16) DW Spectrum analog-to-IP encoder licenses

      - DW Spectrum I/O Module licenses

        - DW-SPIOLSC001		Single (1) DW Spectrum I/O module license

      - DW Spectrum Video Wall licenses

        - DW-SPVWALL1X2		Single (1) DW Spectrum video wall license

  - DW SPECTRUM OVERVIEW

    - DW Spectrum Software Components.

      - The System shall be comprised of four (4) applications that work together to form a seamless System.

        - DW Cloud – A cloud-based service that enables simple remote connectivity, viewing, configuration and management of connected Systems and Users.

          - The Cloud service should be an optional component that is free to use.

        - DW Spectrum Server – A lightweight, high-performance cross-platform service responsible for discovering devices, recording streams, streaming video and other data, and enabling secure connections between clients and connected devices, system resources, and associated metadata. The server shall contain an embedded Web administration interface (compatible with all major browsers).

          - The Server application should be able to run on Windows and Ubuntu Linux.

          - The Server install package should be <200 MB in size and require no prerequisite software (e.g. MySQL, SQL, Visual Studio, .Net, etc) to install and use.

        - DW Spectrum Client – A lightweight, high-performance, cross-platform desktop application capable of acting as a stand-alone media player, a connected client used to actively configure, manage, and view devices and related data, or a video wall / remote monitor.

          - The Desktop Client application should be able to run on Windows, MacOS, and Ubuntu Linux devices.

          - The Desktop Client application installation package should not exceed 200 MB in total size and require no prerequisite software to install and use on supported operating systems.

        - DW Spectrum Mobile – a mobile application for iOS and Android devices that allows users to connect to, view, search, and control IP cameras over Wi-Fi or Data.

          - The Mobile Client application should be able to run on iOS and Android devices.

          - The Mobile Client application installation package should not exceed 100 MB in total size and require no prerequisite software to install and use on supported operating systems.

    - DW Spectrum Developer and Integration Tools.

      - The DW Spectrum shall have built-in developer tools that are accessible from any System Server's Web Admin Interface (compatible with all major browsers) and should include, at a minimum:

        - A Generic Events Generator - a tool that helps build HTTP Generic Event calls, a method of sending events from 3rd party systems to the DW Spectrum, which can be used to trigger system actions in the DW Spectrum.

        - Server/Cloud API – A REST HTTP(S) API that gives developers the ability to develop integrations to 3rd party devices and software using coding languages and frameworks of their choice.

          - System API

          - Server API

          - Cloud API

          - Video API

          - Audio API

          - Proxy API

          - WebSocket API

          - Authentication & Encryption

          - Breaking Change Log

        - Client (JavaScript API) – Allows developers to integrate the web applications running in the embedded browser into a System.

        - API Change Log – list of breaking changes in API from version to version.

        - Video Source Integration SDK – provides the ability to integrate virtually any live or recorded video source (IP Cameras, NVRs, DVRs, etc.) into the DW Spectrum with methods for discovering, displaying, analyzing and recording video, as well as integrating device I/O ports and related motion detection information.

        - Storage SDK – provides the ability to integrate potential storage into the System. It allows developers to read from or write to any storage location: local, remote, and even cloud. Creating a storage plugin requires implementing standard functions such as I/O stream, if a file exists, delete file, list of files in the folder, etc. Storage SDK also contains an example of using an FTP server as a storage location.

        - Metadata SDK – provides the ability to rapidly integrate 3rd party video and data analytics solutions by creating and installing Plugins on the Server application. Capabilities include requesting frames from the DW Spectrum in yuv420, argb, abgr, rgba, bgra, rgb and bgr formats, providing object size and coordinates, and returning metadata and events, which can then be displayed in the System's client interfaces.

    - System Architecture

      - The DW Spectrum shall have a Server Hive Architecture wherein:

        - All servers in a system are equal and synchronize system databases in real time.

        - A user can connect to any system server to see and manage the entire System.

        - Servers support automatic camera fail-over to ensure a limited loss of video recording in the event of hardware or network failure.

        - Servers will use SQLite - a database technology - included in the installation package and will require no prerequisite software other than a supported operating system.

      - The DW Spectrum shall support one-click system-wide updates.

        - System Administrators shall be able to upgrade an entire system via a single button in the Desktop Application.

        - System Administrators shall be able to upgrade on demand to the latest release or specific builds with specific functionality or bug fixes.

        - System Administrators shall be able to apply an OTA (over-the-air) update.

        - System Administrators shall be able to generate a URL to download a portable system-specific update package in .zip file format, which can be used to update servers without an active Internet connection.

      - The DW Spectrum will use secure technologies for inter-application communication and security.

        - OpenSSL for network connections - deprecated and insecure protocols and use only TLS v1+.

        - Server -> Client (Mobile, Desktop, Web) Communications - HTTPS Email - TLS / SSL - TLS is the default option for the Email Server.

        - Option to force HTTPS video traffic encryption between Client and Server.

        - HTTPS Email notification - TLS / SSL - TLS is the default option for Email Server communications.

        - Salted/Hashed Passwords - Local Credentials will be protected using a salted MD5 hash, and Cloud Credentials should use a complex multi-level hash.

        - Encrypted video and communications between DW Spectrum Server and Client (Desktop, Mobile, Web) interfaces.

      - The DW Spectrum will not require any licenses to increase the number of supported devices, users, or servers or to support automatic camera fail-over.

  - DW SPECTRUM SERVER APPLICATION

    - DW Spectrum Server Supported Operating Systems:

      - Microsoft® Windows®:

        - Windows 10

        - Windows 11

        - Windows Server 2016

        - Windows Server 2019

        - Windows Server 2022

      - Ubuntu® Linux®:

        - Ubuntu 20.04 LTS

        - Ubuntu 22.04 LTS

    - DW Spectrum Server Application Minimum Compatible / Supported Computing Hardware.

      - The DW Spectrum will be capable of running on any computing hardware able to run a compatible operating system, and that has, at a minimum:

        - 1GB of RAM

        - 100 Mbps NIC

        - Intel® Dual-Core Atom Processor or equivalent compatible CPU

      - The DW Spectrum Server application will be written in such a way that it will be capable of running 128 dual-streaming IP cameras (256 total streams) on a single core of an Intel® Core i3 CPU.

    - DW Spectrum Server Application Initial Installation and Setup.

      - The DW Spectrum Server application should be installed without the need for any prerequisite software (proprietary databases or 3rd party software) other than a supported operating system.

      - The DW Spectrum Server installer package (and any dependencies, if needed) should not exceed 100 MB (megabytes).

      - The DW Spectrum installation process should require no user input once initiated.

      - After installation is complete, the DW Spectrum Server setup process will allow system administrators to create a new system or merge the newly installed server(s) with an existing system.

    - DW Spectrum Server Application Features

      - The DW Spectrum Server Application shall automatically discover, stream, and record any ONVIF Profile S IP camera or encoder located on the same subnet as the server application and will also discover and stream video and audio from connected USB Webcams, compatible I/O devices, and compatible Audio devices.

      - The DW Spectrum Server Application shall allow administrators to manually discover, stream, and record RTSP, HTTP, or UDP (multicast, unicast) streams.

      - The DW Spectrum Server application shall support up to 1000 concurrent TCP connections.

      - The DW Spectrum Server application shall record and stream video of any resolution and frame rate, limited only by hardware.

      - The DW Spectrum Server application shall support automatic camera fail-over without any additional licenses.

      - The DW Spectrum Server application will support an unlimited number of users and custom user roles.

      - The DW Spectrum Server application shall support any storage medium - HDDs, SSDs, SD cards, DAS, NAS, or other network-attached storage devices or locations.

      - The DW Spectrum Server application shall support LDAP / Active Directory / Open LDAP integration for user login credential management.

      - The DW Spectrum Server application shall record and stream H.264, H.265, and MJPEG streams.

      - The DW Spectrum Server application shall record and stream AAC, PCM (Mu-Law, 
A-law), g726, and MP3 audio.

      - The DW Spectrum Server application shall support transcoding streams on-demand for delivery to 3rd party systems or devices in H.265, H.264, MJPEG or WebM codecs.

      - The DW Spectrum Server application shall be able to provide pass-through high or low-res HLS streams from connected devices.

      - The DW Spectrum Server application shall store archive indices in the same location as recorded video files.

      - The DW Spectrum Server application shall allow system administrators to recover archives from any storage medium using a reindex archive feature.

      - The DW Spectrum Server application will contain a Boolean events engine allowing operators to program and trigger system actions based on System, connected device, or HTTP events sent from 3rd party system or device.

      - The DW Spectrum Server application shall be able to send HTTP PUT or GET requests to 3rd party systems or devices.

      - The DW Spectrum Server application shall support IPv4 or IPv6 address

      - The DW Spectrum Server application shall allow operators to set custom network routing configurations for system servers to optimize network routing and usage.

      - The DW Spectrum Server application shall allow operators to monitor the CPU, RAM, NIC, and HDD usage in real time.

      - The DW Spectrum Server application shall track all operator actions to allow audits.

      - The DW Spectrum Server application shall generate automatic crash files every time there is an unexpected crash of the Server application.

      - The DW Spectrum Server application shall allow operators to change the size of reserved disk space for storage drives.

      - The DW Spectrum Server application shall automatically turn off any system drive (drive containing the operating System) in computing hardware with more than one drive to ensure the operating system drive does not become full.

      - The DW Spectrum Server application shall support configuration and events from binary I/O contacts on supported devices - including IP cameras and I/O devices.

      - The DW Spectrum Server application shall support sending email notifications via SMTP using TLS, SSL or unsecured connections.

      - The DW Spectrum Server application shall support scheduled backup of recording archives to local, networked, or cloud storage locations.

      - The DW Spectrum Server application shall allow on-demand backup of recording archives to local, networked, or cloud storage locations.

      - The DW Spectrum Server application shall allow concurrent recording of all connected cameras/streams to two (2) servers in real time.

      - The DW Spectrum Server application will allow server-side, CPU-based motion analysis for all connected IP cameras with no perceptible increase (<3%) in CPU usage.

      - The DW Spectrum Server application will require no dedicated GPU to perform at maximum capacity.

      - The DW Spectrum Server application will have a web administration interface that allows users to view live or recorded video from a single camera at a time in high or low resolutions.

      - The DW Spectrum Server application will have a web administration interface that allows system administrators to view real-time server health monitoring statistics (CPU, NIC, and 
HDD usage).

      - The DW Spectrum Server application will have a web administration interface that allows operators to disconnect the DW SPECTRUM Server from the DW SPECTRUM cloud application.

      - The DW Spectrum Server application will have a web administration interface that allows users to view all available servers in the System.

      - The DW Spectrum Server application will have a web administration interface that allows operators to switch between server interfaces.

      - The DW Spectrum Server application will have a hidden advanced page that gives system administrators the ability to modify advanced system settings.

      - The DW Spectrum Server application will support any RAID configuration of the storage medium.

      - The DW Spectrum Server will support simple discovery and addition (authenticated or unauthenticated) of SAMBA NAS devices.

      - The DW Spectrum Server application will allow administrators to configure custom network routing rules for the System to optimize for limited network environments.

      - The DW Spectrum Server application will be able to send and receive custom HTTP or HTTPS events for use in syntactic integrations with 3rd party hardware and software.

  - DW SPECTRUM DESKTOP APPLICATION

    - DW Spectrum Desktop Application Supported Operating Systems:

      - Microsoft Windows

        - Windows 8.1

        - Windows 10

        - Windows Server 2012

        - Windows Server 2012 R2

        - Windows Server 2016

        - Windows Server 2019

        - Windows Server 2022

      - Ubuntu Linux

        - Ubuntu 18.04 LTS

        - Ubuntu 20.04 LTS

        - Ubuntu 22.04 LTS

      - Apple MacOS

        - macOS 11, 11.2, 11.3

        - macOS 12

        - macOS 13

    - DW Spectrum Desktop Application Minimum Compatible / Supported Computing Hardware:

      - The DW Spectrum will be capable of running on any computing hardware able to run a compatible operating system, and that has, at a minimum:

        - a CPU that supports OpenGL 2.1

        - Intel® HD Graphics 3000

      - The DW Spectrum Desktop application shall not require any dedicated graphics card to work at full capacity (64 streams on a 64-bit OS). It shall use the CPU for all video decoding and rendering.

    - DW Spectrum Desktop Application Initial Installation and Setup:

      - The DW Spectrum Client application installer should not exceed 100 MB (megabytes).

      - The DW Spectrum Client application should be publicly available and available for free download.

      - The DW Spectrum Client application should require no prerequisite proprietary or 3rd party software and database technologies during installation.

      - The DW Spectrum Client installation process should require no user input once initiated.

    - DW Spectrum Desktop Application Features:

      - The DW Spectrum Desktop application will have the following basic structure:

        - Navigation Panel - with a main menu button, an interactive cloud-login icon, tabbed layouts, minimize and maximize icons, a contextual help icon, and a close application.

        - Resource Panel (Left) - contains all system resources (Servers, Devices, Users, Layouts, Offline files, etc.) with a collapsible structure and a keyword search mechanism to allow operators to quickly search for a display live streams/cameras, offline video and image files, or any combination.

        - Notifications Panel (Right) - shows all System- or rules-engine-generated notifications that can be clicked on to display relevant resources in the Viewing Grid.

        - Timeline / Playback Panel (Bottom) - allows for navigation and search of recorded video files.

        - Viewing Grid (Main Viewing Area) - a flexible, adaptive grid interface that allows operators to create and share customized layouts of the System.

        - Advanced Object Search Dialog - the VMS should allow operators to search for detected objects by type and attributes.

      - The DW Spectrum Desktop application shall allow operators to view and interact with the following types of media:

        - Live Streams: H.265, H.264, MJPEG.

        - Offline Media: AVI MKV MP4 MOV TS M2TS MPEG MPG FLV WMV 3GP JPG PNG GIF BMP TIFF.

        - I/O Devices: Status and Triggers.

        - Servers: Real-Time Server Health Monitoring Status.

      - The DW Spectrum Desktop application shall allow the operator to scroll to zoom in on any part of the Viewing Grid.

      - The DW Spectrum Desktop application shall allow the operator to drag & drop to reassign cameras from one server to another server.

      - The DW Spectrum Desktop application will have a flexible timeline that allows operators to view the dates of any and all archived video in the System for a specific camera or groups of cameras.

      - The DW Spectrum Desktop application will allow operators to manually create bookmarks - with a start time, end time, name, description, and tags - for later search. Bookmarks can also be created using the Rules Engine.

      - The VMS Desktop application should allow operators to create automation using the Rules Engine with the following Events & Actions:

        - Events

          - User-Defined Events: Custom Events which must be programmed by a user before they can be utilized and include:

            - Analytics Event

            - Analytics Object Detected

            - HTTP Generic Event (covered in Advanced Training)

            - Input Signal on Device

            - Motion on Camera

            - Plugin Event

            - Soft Trigger

          - Default Events: Configurable Events populated in the Rules Engine when a new DW Spectrum System is installed.

            - Device Disconnected

            - Device IP Conflict

            - License Issue

            - Network Issue

            - Server Conflict

            - Server Failure

            - Server Started

            - Storage Issue

          - System Generated Events: built-in Events in which the System will generate a notification to operators and cannot be modified by System users.

            - Archive Integrity Check Failure

            - Email Address Not Set

            - Email Not Set for Users

            - Email Server Not Configured

            - Error while Sending Email

            - LDAP Sync Issue

            - Licenses Not Configured

            - Local storage is used for analytic and motion data

            - Reindexing Archive Canceled

            - Reindexing Archive Complete

            - Remote Archive Synchronization

            - Storage not Configured

            - System in Safe Mode

            - Time Synchronization Issue

            - Server Certificate Error

        - Actions

          - Bookmark

          - Device Output

          - Do Recording

          - Do HTTP Request

          - Execute PTZ Preset

          - Exit Fullscreen

          - Open Layout

          - Panic Recording

          - Play Sound

          - Repeat Sound

          - Send Email

          - Set to Fullscreen

          - Send Mobile Notification

          - Send Desktop Notification

          - Show on Alarm Layout

          - Show Text Overlay

          - Speak

          - Write to Log

      - The DW Spectrum Desktop application shall allow operators to create Soft Triggers - programmable, customizable buttons that sit on top of streams in the Viewing Grid - to trigger any available system action.

      - The DW Spectrum Desktop application shall have icons located on the top of live camera streams, which allow operators to dewarp fisheye cameras, control PTZ cameras, apply client-side image enhancement, execute smart motion search, create zoom windows, rotate items to any orientation, and activate stream or file info.

      - The DW Spectrum Desktop application shall allow operators to create Zoom Windows (up to 63 zoom windows on a single item in a 64-bit OS) - a magnified view of a part of a live stream, recorded videos, or static images.

      - The DW Spectrum Desktop application shall allow operators the ability to execute a Smart Motion search by selecting a subset of a live camera stream with results shown in red on the flexible timeline. Smart Motion search should be able to search a year (12 months, 365 days) of archived video in less than one (1) second.

      - The DW Spectrum Desktop application will allow users to search live cameras by name, Manufacturer, IP address, MAC address, and status (e.g., live).

      - The DW Spectrum Desktop application shall allow operators to search video archives by date and time with a responsive, adaptive timeline.

      - The DW Spectrum Desktop application will allow operators to customize the background image of the application with supported image types.

      - The DW Spectrum Desktop application will support digital mapping by allowing operators to add and customize background images - including opacity and number of grid items to be displayed by default.

      - The DW Spectrum Desktop application will utilize adaptive scaling technology to automatically switch between high and low-resolution streams during live and recording playback to optimize CPU and network usage.

      - The DW Spectrum Desktop application will allow operators to log in to the Cloud application in order to connect to any shared system quickly.

      - The DW Spectrum Desktop application will allow operators to quickly switch between previously connected or cloud-connected systems using searchable tiles that show the system name and status.

      - The DW Spectrum Desktop application will have a Storage Analytics feature allowing operators to analyze the storage capacity of the System based on available drives and real-time and historical bandwidth analysis.

      - The DW Spectrum Desktop application will allow management and configuration of all System devices, users, and resources in a single unified interface.

      - The DW Spectrum Desktop application will allow fast-forward and fast-reverse archived video up to 16x normal speed.

      - The DW Spectrum Desktop application will show operators which system server they are connected to.

      - The DW Spectrum Desktop application will allow operators to connect to previous versions by automatically downloading and switching to compatible versions.

      - The DW Spectrum Desktop applications will automatically discover available systems on the same network as the computer running the Desktop application.

      - The DW Spectrum Desktop application will automatically recover and reconnect to a system if the server the operator is connected to becomes inaccessible for any reason.

      - The DW Spectrum Desktop application will allow operators to show or hide adaptive thumbnails in the timeline/playback panel.

      - The DW Spectrum Desktop application will allow operators to synchronize all items on a layout or turn off synchronization to view live and recorded video at the same time.

      - The DW Spectrum Desktop application will have an adaptive resolution settings dialog, allowing operators to switch resolution settings while the dialog is open by clicking on Auto (Default),  High or Low. a

      - The DW Spectrum Desktop application will allow batch configuration of camera recording schedules, fps and quality.

      - The DW Spectrum Desktop application will allow operators to drag and drop multiple system resources onto the Viewing Grid at the same time.

      - The DW Spectrum Desktop Application will allow administrators to modify time synchronization settings for the System to utilize online resources (NTP servers) or to set up a dedicated local time server.

      - The DW Spectrum Desktop Application will allow system administrators to view a full list of system cameras and devices in a single dialog.

      - The DW Spectrum Desktop application will allow operators to view, search and export all system events.

      - The DW Spectrum Desktop application will allow operators to view, search and export all system bookmarks.

      - The DW Spectrum Desktop application will allow operators to view, search, and export system logs.

      - The DW Spectrum Desktop application will allow operators to view, search, and export an audit trail of all operator actions and replay related videos.

      - The DW Spectrum Desktop application will allow administrators to backup and restore the system database.

      - The VMS Desktop application will allow administrators to replace a camera while preserving the archive from the previous one.

      - The VMS Desktop application will provide predefined user groups.

      - The VMS Desktop application will provide guests with temporary access to the system with limited permissions.

      - The DW Spectrum Desktop application will allow administrators to create an unlimited number of custom user roles.

      - The DW Spectrum Desktop application will allow administrators to create and share lockable layouts.

      - The DW Spectrum Desktop application will allow administrators to update layouts in real time.

      - The DW Spectrum Desktop application will allow users to record their screen in full resolution and up to 30 fps.

      - The DW Spectrum Desktop application will allow users to add a local folder to add local files for search and playback.

      - The DW Spectrum Desktop application will have a Video Wall mode, which will allow operators to control the application remotely.

      - The DW Spectrum Desktop application will have a Media Player mode which will allow operators to use the application as a media player.

      - The DW Spectrum Desktop application will remember past system connections and user credentials and will allow operators to search for and switch between systems quickly.

      - The DW Spectrum Desktop application will allow operators to adjust the aspect ratio and streaming quality (high resolution or low resolution) of items displayed on the viewing grid.

      - The DW Spectrum Desktop application will display I/O devices as an individual item on the viewing grid and allow operators to create custom names for inputs and outputs.

      - The DW Spectrum Desktop application will allow users to customize the layout of I/O panels on the item in the viewing grid, including indicators for inputs and buttons for outputs.

      - The DW Spectrum Desktop application will allow users to dewarp any fisheye lens using automatic calibration or manual calibration without the need for any 3rd party SDKs.

      - The DW Spectrum Desktop application will allow users to create fully customizable viewing tours that include any combination of live video streams, offline videos, images, websites (or URLs), I/O devices, and server health monitoring status.

      - The VMS Desktop application will allow users to create fully customizable viewing tours that include any combination of live video streams, offline videos, images, websites (or URLs), I/O devices, and server health monitoring status.

      - The DW Spectrum Desktop application will allow system administrators to modify and save a shared layout to affect an instantaneous change to that layout on the 
DW Spectrum Desktop application is for any user connected to the system viewing that layout (when the system administrator saves the layout, it will update in real-time for any user viewing that layout).

      - The DW Spectrum Desktop application will support two-way audio between operators and supported devices.

      - The DW Spectrum Desktop application will support audio alerts as an action that can be played on users' computers or connected system devices.

      - The DW Spectrum Desktop application will support PTZ presets and PTZ tours.

      - The DW Spectrum Desktop application will support PTZ presets and tours in fisheye cameras using dewarp mode.

      - The VMS Desktop application will support object following in real-time through different cameras (Camera Hotspot and Camera Linking)

      - The DW Spectrum Desktop application will allow operators to schedule a recording for connected cameras and devices with options to force a minimum or maximum storage duration.

      - The DW Spectrum Desktop application will allow operators to configure pre and post-recording for motion events.

      - The DW Spectrum Desktop application will allow operators to optimize camera streaming quality from connected devices automatically using low, medium, high, and best quality selectors or manually in the camera.

      - The DW Spectrum Desktop application will allow users to export video by selecting a camera or cameras, then selecting an area on the timeline, right-clicking, and choosing export in the context menu.

      - The DW Spectrum Desktop application will support single video export in .avi, .mp4, or .mkv formats. It will offer the option to transcode any client-side effects (image enhancement, dewarping and timestamps) as part of exported video.

      - The DW Spectrum Desktop application will support multi-video export in an executable format to create a fully portable version of the DW Spectrum Desktop application, including all exported video files.

      - The DW Spectrum Desktop application shall have a rapid review export feature that will allow operators to compress any length of video into a short video (e.g., export 8 hours of archives into a 30-second video clip).

      - The DW Spectrum Desktop application shall allow system administrators to activate or deactivate system licenses on Internet-connected systems.

      - The DW Spectrum Desktop application shall allow users to force open an alarm layout triggered by any system or 3rd party event with one or many associated cameras or system resources (including web pages, images, offline videos, I/O devices, etc.).

      - The DW Spectrum Desktop application will have a hidden configurable method of increasing the number of items allowed on the viewing grid.

      - The DW Spectrum Desktop application shall allow users to adjust the configuration of connected IP video devices (IP cameras, I/O devices, encoders, etc.).

      - The DW Spectrum will now allow analytics from DW®, Axis, Hikvision, Dahua, and any device supporting ONVIF Analytics Events.

      - The DW Spectrum Desktop application will force users to set the camera's initial password upon enrollment for optimal cybersecurity.

      - The DW Spectrum Desktop application will allow administrators to configure installed plugins for 3rd party video analytics solutions and display detected objects in the Notifications panel.

      - The VMS Desktop application should allow operators to access a camera web page as part of the Camera Settings dialog both locally and remotely (proxied via VMS) with a dedicated browser window embedded in the Camera Settings Dialog.

      - The VMS Desktop will allow users to enable client-side Intel Quicksync decoding.

      - The DW Spectrum Desktop application will allow administrators to configure the security of system connections (HTTP, HTTPS) and enable encrypted video and communications between the Server application and system clients (Desktop, Mobile, Web).

      - The DW Spectrum Desktop application will support advanced PTZ for supported cameras that allow operators to draw a rectangle to zoom to a specific point of interest and to double-click to zoom to the original zoom parameters.

  - DW SPECTRUM MOBILE APPLICATION

    - DW Spectrum Mobile Application Supported Operating Systems:

      - Google Android

        - Android 9

        - Android 10

        - Android 11

        - Android 12

        - Android 13

        - Android 14

      - Apple iOS

        - iOS 16

        - iOS 17

        - iPadOS 16

        - iPadOS 17

    - DW Spectrum Mobile Download and Installation:

      - The DW Spectrum Mobile application will be available for free download from Google Play or Apple iTunes stores.

    - DW Spectrum Mobile Features:

      - The DW Spectrum Mobile application will automatically discover available Systems on a local area network (LAN).

      - The DW Spectrum Mobile application will store past system connections and credentials and will allow users to search for switches between systems quickly.

      - The DW Spectrum Mobile application will have adaptive streaming and automatically adjust the stream being displayed based on network speed.

      - The DW Spectrum Mobile application will allow users to adjust streaming resolutions manually.

      - The DW Spectrum Mobile application will allow users to search for cameras by name.

      - The DW Spectrum Mobile application will allow fisheye dewarping of any fisheye lens without the need for any 3rd party SDK.

      - The DW Spectrum Mobile application will allow users to view live video from one System.

      - The DW Spectrum Mobile application will allow users to log in to the DW Spectrum Cloud layer in order to view and access all systems shared with a user.

      - The DW Spectrum Mobile application will utilize a custom media player to render and display live thumbnails and videos.

      - The DW Spectrum Mobile application will allow users to search for videos using a calendar.

      - The DW Spectrum Mobile application will allow users to search for videos using a flex timeline.

      - The VMS Mobile application shall support Push Notifications when connected to the DW Cloud.

      - The VMS Mobile application should support two-way audio

      - The VMS Mobile application should have live thumbnails for connected cameras in a multi-camera view.

      - The VMS Mobile application should allow users to control PTZ cameras with a single touch-to-move.

      - The VMS Mobile application should allow users to navigate through bookmarks.

      - The VMS Mobile application should support Push Notifications, which can be configured separately for each system in a multi-system environment.

      - The DW Spectrum Mobile application will allow users to search for cameras by layout.

      - The DW Spectrum Mobile application will allow users to control devices and system actions with Soft Triggers.

      - The DW Spectrum Mobile application will allow users to search using Smart Motion search by selecting all or part of a camera view.

  - DW CLOUD APPLICATION

    - DW Spectrum Cloud Supported Browsers:

      - The DW Cloud application will allow users to log in from any modern web browser (Google® Chrome, Mozilla® Firefox, Microsoft® Edge, Opera®, etc.) from any device (mobile, pc, etc.).

    - DW Cloud Connection Setup:

      - The DW Cloud application will allow System Owners to connect an unlimited number of Systems to a single user account.

      - The DW Cloud application will allow any System with an active internet connection to connect to the DW Cloud service, which will then make the System accessible from anywhere, anytime without the need for port forwarding or dynamic DNS services.

    - DW Cloud Features:

      - The DW Cloud application will be an optional add-on to the DW Spectrum, requiring no additional licensing.

      - The DW Cloud application will allow users to connect an unlimited number of systems to a single user account.

      - The DW Cloud application will allow system administrators to share access to a system using only an email address.

      - The DW Cloud application will allow system administrators to assign custom user roles when sharing system access.

      - The DW Cloud application will allow users to search for and connect to cloud-connected systems by name quickly.

      - The DW Cloud application will allow operators to view live or recorded video from one camera at a time on any cloud-connected system.

      - The DW Cloud application will first attempt a direct connection to system servers using NAT Traversal technology. It will be able to proxy traffic to ensure access to a system in the case of ISP or routing issues.

      - The DW Cloud application will allow an unlimited number of connected users and systems with no additional licensing.

      - The DW Cloud application will utilize secure networking technologies (OpenSSL, HTTPS) and a complex Salted MD5 hash for any stored passwords.

      - The DW Cloud application will include a web client that can be used to search for and view cameras in the browser.

      - The VMS Cloud application will allow two systems to be merged to operate as one system without the need for port forwarding or local access.

      - The DW Cloud application should have a Health Monitoring Dashboard that shows all system devices, their status, and associated metrics, as well as the ability to download a JSON log file containing all health information from the past 24 hours.

      - The DW Cloud application will allow operators to view and modify camera settings, including aspect ratio, rotation, audio, authentication, recording settings, quality of recording, and motion detection sensitivity.

      - The DW Cloud application will allow operators to view, restart, rename, or change communication port settings for all Servers in a System.

      - The DW Cloud application should allow operators to enable and force two-factor / OAUTH2 authentication for all users in a System.

      - The DW Cloud application should allow operators to export video archive portions.

      - The DW Cloud application should provide two different skins (modes): Light and Dark.

      - The DW Cloud application should allow system owners to transfer ownership to another user.

  - MINIMUM HARDWARE REQUIREMENTS

    - Server

      - Up to 16 cameras:

        - Processor:				Intel® Quad Core Celeron or higher

        - System RAM:			4 GB

        - Acceptable Operating Systems:

          - Microsoft® Windows® 7/8/10

          - Microsoft® Windows® Server 2012

        - Network Interface: 			Dual Gigabit (RJ-45)

      - Up to 64 cameras:

        - Processor:				Intel® i5 or higher

        - System RAM:			8 GB

        - Acceptable Operating Systems:

          - Microsoft® Windows® 7/8/10

          - Microsoft® Windows® Server 2012

        - Network Interface: 			Dual Gigabit (RJ-45)

      - Up to 128 cameras:

        - Processor:				Intel® i7 or higher

        - System RAM:			16 GB

        - Hard drive (HDD) storage:		60 GB SSD, 7200 rpm HDD

        - Network Interface:			Dual Gigabit (RJ-45)

        - Acceptable Operating Systems:

          - Microsoft® Windows® 7/8/10

          - Microsoft® Windows® Server 2012

        - Network Interface: 			Dual Gigabit (RJ-45)

    - Client

      - Processor:				Intel® i3 4th Generation

      - Video

        - Graphics card:			Intel® HD graphics or comparable with 1GB memory

      - RAM:					4 GB

      - Storage:				500 GB HDD

      - Network Interface:			1Gbps (RJ-45)

      - Acceptable Operating Systems:

        - Microsoft® Windows® 7/8/10

        - Microsoft® Windows® Server 2008/2012

        - Mac® OS X

      - Mobile Client:

        - Acceptable Operating Systems

          - Google® Android

          - Apple® IOS

        - Mobile clients shall be readily available through device App Stores.

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

    - Contractor personnel shall be factory-trained and certified to install, service and maintain the System.

  - PREPARATION

    - Camera and encoder compatibility shall be verified prior to deployment.

    - The network design and configuration shall be verified for compatibility and performance with the 
DW Spectrum.

    - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

  - INSTALLATION

    - The Contractor shall follow all Manufacturer published installation procedures and guidelines.

    - Servers shall be installed in areas consistent with Manufacturer recommendations.

END OF SECTION